Are we properly assessing bird and bat mortality at onshore wind farms?

Introduction

Wind farms can cause negative impacts on bird and bat communities, such as habitat displacement and direct mortality. Over the years, high fatality rates have been associated to some wind energy facilities, which raised concerns about wind energy’s real impacts on birds and bats. In response, the development of methods to properly estimate mortality and assess impacts on populations has become a priority.

Several factors can influence the reliability of the mortality estimates and therefore must be taken into account during the definition of the experimental design and the selection of the methodologies to adopt. At the same time, due to financial limitations, it is crucial to develop and validate field protocols that reduce the costs without compromising the estimates accuracy and precision.

Carcass searches

At onshore facilities the estimation of bird and bat mortality is generally based on carcass searches around wind turbines. Within a pre-defined plot, trained searchers look for carcasses walking, for instance, along parallel transects. Inspections are then repeated over time at regular (or irregular) intervals depending on the purpose of the study. However, the number of carcasses found during the searches does not correspond to the real number of birds and bats killed by the wind turbines. This value must be adjusted by carcass removal (e.g. by scavengers or decay) and searcher efficiency rates, which can in turn be influenced by site- and carcass-specific covariates, such as carcass size, season, vegetation cover and orography.

Carcass persistence rates

Current post-construction monitoring protocols accommodate carcass removal trials to assess the carcasses persistence. Trials consist of placing carcasses under the turbines that are then checked for a pre-defined period of time. The experimental design should take into consideration the different factors that can influence the removal rate (such as season and carcass size) and the minimum number of carcass that must be placed to ensure the statistical robustness of the trials. Additionally, to avoid scavenger swamping, it is important to remember that only a limited number of carcasses should be placed simultaneously (Smallwood 2007).

Once placed, different protocols regarding the frequency and interval between the checks may be adopted to determine the average time of persistence of each carcass. At remote wind farms, frequent checks (e.g. daily) may imply a significant financial effort. Therefore it’s essential to fully understand the impact that a reduction on the survey effort may have on the accuracy of the estimated correction factor. Bernardino et al. (2011) evaluated the effect of different inspection protocols on the estimation of the carcass persistence rate at 3 Portuguese wind farms. In this study we compared the results of field trials, that consisted in daily checks for a period of 20 days, with 4 other hypothetical scenarios: i) daily checks for 14 days, ii) daily checks for 7 days, iii) checks every other day for 20 days and iv) daily checks in the first 4 days and then in the 7th, 10th, 14th and 20th days. The statistical analysis evidenced significant differences between the removal correction factors whenever the survey effort was reduced. The only exception was when the trial length was reduced from 20 to 14 days (see Bernardino et al. 2011, for more details).

This study highlighted not only the impact that field protocols may have on estimates uncertainty but also the importance of selecting the best method to determine the scavenging removal correction factor. Up to that point, it had been determined using either observed proportions or the mean time of removal over a period of time. Yet we have proposed a new statistical approach for the analysis of the time of carcass removal, using parametric survival analysis (Bispo et al. 2010). Through this method, different lifetime distributions (e.g. exponential, Weibull, log-logistic and log-normal) may be fitted to data and the model fit analyzed using both adequacy measures (AIC) and graphical analysis. Ultimately carcass persistence rates should be estimated upon the best-fitted model.

Carcass detectability

Carcass searches are traditionally performed by human observers, which efficiency is influenced by carcass-specific characteristics (size, coloration and state of decomposition), site-specific characteristics (e.g. vegetation cover, orography, weather conditions) and observer-specific characteristics (e.g. ability to detect, commitment). To assess the carcass detection rate at a particular study area, searcher efficiency trials must be conducted, typically by placing carcasses (or surrogates) under turbines and calculating the average percentage of found carcasses. Depending on the available resources and monitoring objectives, the experimental design of the trials may or may not include covariates.

The trials carried out worldwide evidenced that in some cases human searchers can present low efficiency rates, which may contributes to severe bias in mortality estimates. Thus, the use of dogs and their olfactory capabilities has been suggested as a way to increase carcass detection rates (Homan et al. 2001, Arnett et al. 2006).

We conducted a study that evidenced the usefulness of using dogs during field surveys to improve bird-strike mortality estimates at wind farms and other anthropogenic structures (Paula et al. 2011). During a 3 months period, two dog-handlers teams were trained until dogs were able to stay point and bark in the presence of a bird or bat carcass. After this intensive period of training, trials were performed in order to assess (1) if dogs are more accurate than humans to detect bird and bat carcasses under different vegetation conditions, and (2) if carcass decomposition, weather conditions (temperature and wind speed) and distance to the target affects the search accuracy and efficiency of the working dog. Results showed that while dogs detected 96% of the carcasses placed, human searches only found 9%. Dogs’ accuracy was independent of vegetation density and the effects of carcass decomposition state, distances to the carcass and weather conditions on dogs’ efficiency were minor.

Fatality estimates

The final purpose of conducting carcass searches and estimating the associated bias correction factors (carcass persistence and detectability rates) is to calculate the number of fatalities per year and single turbine or, alternatively, per MW. Over the years several estimators have been proposed (e.g. Johnson et al. 2003, Erickson et al. 2004, Kerns et al. 2005, Jain et al. 2007, Huso 2010, Korner-Nievergelt et al. 2011), mainly for wind power facilities but that can be applied to other human infrastructures. However, some of these mortality estimators are complex, requiring, in some cases, a great effort and statistical expertise from the technician who is conducting the data analysis. The same applies to the methods that can be used to estimate the correction factors.

To overcome that we have developed the web-based application “Wildlife Fatality Estimator” (www.wildlifefatalityestimator.com) that helps users to properly apply the existing methodologies to estimate mortality and save time for other data analysis. This totally free web tool is divided in 3 application modules:

- Carcass Persistence - section devoted to the estimation of the carcass persistence probability;

- Search Efficiency - section devoted to the estimation of the detection probability;

- Fatality Estimation - section devoted to the final fatality estimation.

With two of the 3 modules fully developed, users can already fit to their data the survivor function that best describes the time until carcass removal, calculate the associated correction factor and then produce mortality estimates, having available 3 estimators (Jain et al. 2007, Huso 2010 and Korner-Nievergelt et al. 2011).

Conclusion

In summary, combining optimized field protocols and adequate statistical methods to estimate mortality and the associated correction factors is fundamental to fully understand and evaluate the real impact of wind farms on birds and bats communities.

References

Arnett, E. B. (2006). A preliminary evaluation on the use of dogs to recover bat fatalities at wind energy facilities. Wildlife Society Bulletin, 34, 1440–1445.

Bernardino J., Bispo, R., Torres, P., Rebelo, R., Mascarenhas, M. & Costa, H. (2011). Enhancing carcass removal trials at three wind energy facilities in Portugal. Wildlife Biology in Practice, 7(2): 1-14.

Bispo, R., Palminha, G., Bernardino, J., Marques, T. & Pestana, D. (2010). A new statistical method and a web-based application for the evaluation of the scavenging removal correction factor. In Proceedings of the VIII Wind Wildlife Research Meeting, Denver, EUA.

Erickson, W.P., Jeffrey, J., Kronner, K. & Bay, K. (2004). Stateline Wind Project Wildlife. Monitoring Final Report, July 2001 – December 2003. Technical report peer-reviewed by and submitted to FPL Energy, the Oregon Energy Facility Siting Council, and the Stateline Technical Advisory Committee.

Homan, J. H., Linz, G., & Peer, B. D. (2001). Dogs increase recovery of passerine carcasses in dense vegetation. Wildlife Society Bulletin, 29, 292–296.

Huso, M. (2010). An estimator of wildlife fatality from observed carcasses. Environmetrics 10 22: 318-329.

Jain, A., Kerlinger, P., Curry, R. & Slobodnik, L. (2007). Annual Report for the Maple Ridge Wind Power Project: Postconstruction Bird and Bat Fatality Study - 2006. Final Report. Curry and Kerlinger, LLC.

Johnson, G., Erickson, W., Strickland, M.D., Shepherd, M.F., Shepherd, D.A., Sarappo, S.A., et al. (2003). Mortality of Bats at a Large-scale Wind Power Development at Buffalo Ridge, Minnesota. American Midland Naturalist, 150, 332–342.

Kerns, J, Erickson, W.P. & Arnett, E.B. (2005). Bat and bird fatality at wind energy facilities in Pennsylvania and West Virginia. In E.B. Arnett (Ed.), Relationships between bats and wind turbines in Pennsylvania and West Virginia: an assessment of bat fatality search protocols, patterns of fatality, and behavioural interactions with wind turbines. A final report submitted to the Bats and Wind Energy Cooperative. Bat Conservation International, Austin, Texas, USA. pp 24–95.

Korner-Nievergelt, F., Korner-Nievergelt, P., Behr, O., Niermann, I., Brinkmann, R. & Hellriegel, B. (2011). A new method to determine bird and bat fatality at wind energy turbines from carcass searches. Wildlife Biology. Volume: 17, Issue: 4.

Paula, J., Leal, M. C., Silva, M. J., Mascarenhas, R., Costa, H. & Mascarenhas, M. (2011). Dogs as a tool to improve bird-strike mortality estimates at wind farms. Journal for Nature Conservation, 19(4): 202-208.

Smallwood, K. S. (2007). Estimating Wind Turbine-Caused Bird Mortality. The Journal of Wildlife Management, 71: 2781–2791. doi: 10.2193/2007-006