ASSESSING METHODS OF ANALYSIS OF ENVIRONMENTAL STUDIES

Introduction

Environmental Impact Assessment (EIA) is the tool of environmental management internationally accepted (Jay et al., 2007). However, there are criticisms about the effectiveness of its procedures (Prado Filho and Souza, 2004). Research has focused on evaluating the effectiveness of environmental impact studies as an indicator of the performance of the EIA process (Badr, Zahran and Cashmore, 2011), and different methodologies are used. But, the method most commonly applied are checklists, produced or modified according to the location where they are used (Gray and Edward-Jones; 2003; Canelas et al., 2005; Pinho, Maia and Monterosso, 2007; Badr, Zahran and Cashmore, 2011).

While the different checklists contain criteria in common, there is lack of studies comparing the results of applying different methods. Thus, this study aims to verify if the use of two different lists provides the same conclusions about the environmental study quality.

Methodology

For this study, two checklists were used:

1. Method A: Environmental Impact Statement Review Package - IAU, Oxford Brookes University (Glasson, Therivel and Chadwick, 2005) comprises 96 criteria grouped into eight sections (Table 1). Once the criteria evaluated, following a general attribute to the section and then to the study;

2. Method B: Guide to Technical Analysis of Environmental Impact Studies (Sánchez, 2008) has seven items assessed for 114 issues (Table 1).

Both checklists were applied to the Environmental Impact Studies (EIS) of the Center for Treatment and Disposal of Waste of Caraguatatuba, Caraguatatuba, São Paulo, Brazil (Revita Engenharia, 2010), which is being analyzed by the Environment Agency of State Sao Paulo. Because the process was in the public hearing phase, this assessment could affect the process of decision-making.

Table 1: Description of Method A and Method B

| METHOD A |  | METHOD B |  |
| --- | --- | --- | --- |
| Criteria | Criteria Description | Criteria | Criteria Description |
| 1: Description of the development | Project information, land requirements, project inputs, residues and emissions. | 1: Introduction | Background of project and study. |
| 2: Description of the environment | Description of the occupied areas and surroundings as well as baseline conditions. | 2: General information | Quality of information and sources of funds, schedule and agreement legal. |
| 3: Scoping, consultation and impact identification | Consultation, relevance of the scoping and methodologies for impact identification. | 3: Characterization of the development | Some adjustments were made, adjusting the 22 questions to the development in question. |
| 4: Prediction and evaluation of impacts | Magnitude and significance of impacts, methodology and data used. | 4: Baseline | It was quoted the Term of Reference (ToR) as the base document for analysis. |
| 5: Alternatives | Evaluation of alternatives. | 5: Evaluation of impacts | Identification, prediction and impact assessment, and risk analysis. |
| 6: Mitigation and monitoring | Description of mitigation measures and effects of mitigation. | 6: Environmental management programs | Environmental management programs and environmental monitoring plan. |
| 7: Non-technical summary | Evaluation of non-technical summary. | 7: Summary and Conclusions | Items of character summary or conclusion of the study. |
| 8: Organization and presentation of information | How the information was organized and presented. | 8: Non-technical summary | Item posted by the group. Are evaluated aspects of language and sufficiency of content. |
| Review Grades | A: Relevant tasks well performad, no important tasks left incomplete. B: Generally satisfactory and complete, only minor omissions and inadequacies. C: Can be considered just satisfactory despite omissions and/or inadequacies. D: Parts well attempted but most, as a whole, be considered just unsatisfactory because of omissions and/or inadequacies. E: Not satisfactory, significant omissions or inadequacies. F: Very unsatisfactory, important task(s) poorly done or not attempted. NA: Not applicable in the context of the EIS or the project. | Review Grades | Complete: All relevant information was presented, no additional information is required. Acceptable: The information presented is not complete, however, the omissions should not hinder the continuation of the decision making process. Inappropriate: The information presented has significant omissions, it is necessary to submit additional information before the decision-making process can proceed. |

Results and discussion

Both approaches were applied independently by two teams, consisting of environmental engineers, biologists and chemists. The results (Table 2) were presented and discussed among experts in the field of law and the environment, without the analysis of a group influenced the other.

According to the descriptions of the methods (Table 1), it was possible to identify differences in the criteria. The contextualization of the project was evaluated in Method B by the criteria "Introduction" and, in Method A, by "Description of the development", which addition brought the project description, what in B was presented in "General Information". In "Characterization of the development", the Method B already included the discussion of alternatives, which was analyzed separately in A ("Alternatives").

In Method B, the baseline was compared to the requirements of the Term of Reference, the same have not found in A. On the other hand, the Method A had a further evaluation of methodologies.

The Method A separated the identification, prediction and assessment of impacts on two criteria (3 and 4), allowing insights into the analysis, while in B, it was valued at "Evaluation of impacts". Likewise, the Method A presented in greater depth analysis of mitigation measures and environmental management programs. In B, because it is a method initially designed for highways, the adaptation of the analysis of the measures and programs to landfill was difficult, restricting the evaluation.

As regards non-technical summary, the approach taken by the methods focus the same points. The structural and textual criteria were specific to A (criterio 8), and B, were analyzed under "General Information" and "Baseline".

Also regarding the application of two methods, it was found that is less subjective to use a more restricted set of criteria, ie, it was most applicable to assign rankings “Complete”, “Acceptable” and “Inappropriate” than the concepts of A to F.

For analysis of the evaluations were considered as topics of comparison steps listed by CONAMA Resolution 01/1986 - Brazilian main rule to guide development of this type of study (CONAMA, 1986).

In the analysis of alternative concepts were assigned "F" and "Inappropriate"; the baseline, "C" and "Acceptable"; the identification, prediction and assessment of impacts, "C" and "D" and "Acceptable"; and mitigation measures, "D" and "Acceptable", respectively by Methods A and B. With respect to non-technical summary, the Method A considered him as "A" and Method B as "Complete". The overall quality of the EIS indicated as only satisfactory because of omissions and/or inadequacies - "C" (Method A) and "Acceptable" (Method B). The discrepancies in results between the methodologies focus on the identification, prediction and assessment of impacts and description of mitigation measures. As noted above, the criteria present in the Method A allowed a more detailed analysis of these topics, explaining the more punitive assessment.

Despite the differences, it was possible to highlight the same strengths and, above all, identify the main problems that can interfere in analysis of the environmental feasibility of the project and decision-making.

Among the strengths are the scoping and non-technical summary. The scope allowed the approach of only aspects relevant and usable in decision-making, adopting what Sánchez (2008) calls a targeted approach. Now the non-technical summary was able to reproduce clearly and objectively the EIS information.

The deficiencies were present in the baseline of the biota, in the methodology for impacts classification, the proposition of mitigation measures and, especially, in the selection of alternatives. Although in the prognosis has been reported the main effects of development on the environment, was not performed for evaluation of secondary and cumulative effects, also found fault Zubair (2001) and Wärnbäc-Rydevika and Hilding (2009). In addition, there was no justification for the weights assigned to the methodology of assessing the magnitude of impacts. Thus, no impact was considered important despite serious interventions are proposed, such as diversion of streams and waterfalls, ecosystem change and disposal of plant and animal species.

As Prado Filho and Souza (2004), the description of mitigation measures was carried out in a very short, which can lead to problems in its implementation.

Finally, the most weakness of the study was the analysis of alternatives. The criteria used for choosing the alternative locacional became clear the importance given to economic aspects (MPF, 2004). In addition, we disregarded the presence of the river system, resulting in impacts that even with the proposed measures are of considerable proportions.

Participation is often critical in the EIA process, having often a merely informative or advisory role (Cortner, 2000) and late occurring without influencing decision-making (Almer and Koontz, 2004). In the case analyzed, it was not possible to determine the role, since the process was in phase hearing. But, somehow is going stakeholder involvement.

Table 2: Results of the evaluation of EIS by Method A and by Method B

|  | METHOD A |  | METHOD B |  |
| --- | --- | --- | --- | --- |
| Criteria | Comments | Grade | Comments | Grade |
| 1 | The description of the project was complete and satisfactory, however, were identified only to the waste phase of operation. | B | Development was contextualized and justified. The presence of gaps in relation to potential regional landfill. | Acceptable |
| 2 | Well defined areas of influence. Satisfactory description of the baseline and appropriate methods of baseline for the physical and anthropic systems. Inadequacy in the survey of the fauna and flora. | C | The study brought the key information listed in the guide. However, there was lack of clarity in defining the scope of the project (local or regional character). | Acceptable |
| 3 | As the study is under public hearing was not possible to assess participation. As the main effects on the environment have been reported and analyzed, the item was considered satisfactory, despite its inadequacies as important impacts be condensed into a single topic and there is no mention of secondary and cumulative effects. | C | The area chosen for the realization of the project will result in the transposition of the main bed of a river basin. The criteria for the determination of this area were only listed without presentation of the methodology of choice. The technological alternatives have no possibilities of arrangements of the landfill, such as duration, form of operation, etc.. | Inappropriate |
| 4 | Despite the clear description of the methodology of the impacts, there were no justifications for the weights assigned, leading to underestimation of the importance of significant impacts as the intervention in hydrography. | D | The baseline were made in accordance with the requirements of ToR. However, there was a problem in the survey of fauna. | Acceptable |
| 5 | Methodology has been presented to the sitting the embankment to be constructed, however, between criteria, none were not related to the presence of water bodies, which negatively influence the choice of area and result in the generation of significant effects. | F | The main impacts were predicted and evaluated according to a methodology. However, this methodology did not consider synergistic effects. | Acceptable |
| 6 | Although the mentioned measures for all impacts, their descriptions were succinct. | D | It pointed the protection systems to be adopted, mitigation of impacts, and plans and monitoring programs. However, the proposals were not sufficiently detailed. | Acceptable |
| 7 | The non-technical summary presented in language appropriate all information present in the EIS. | A | The main features were present. Missed glossary of technical terms used. | Acceptable |
| 8 | The organization and presentation of information was satisfactory, but there was softening of the project interventions and does not show the uncertainties of the study. | C | The non-technical summary was complete and true to the EIS. Easily understood and were able to find the main issues of debate focus of environmental viability of the enterprise concerned. | Complete |
| Overall Grade | C |  | Acceptable |  |

Conclusion

Both methodologies, despite their differences, triggered the same conclusions about the quality and the weakness of the EIS, allowing assessing study quality. This suggests that this type of methodology could be useful for consultants and environmental agencies in improving the quality of environmental studies, and can be applied as a tool for systematic analysis related to impact assessment system of the State of Sao Paulo.

References

Almer H L, Koontz T M. Public hearings for EIAs in post-communist Bulgaria: do they work? Environmental Impact Assessment Review 2004;24:473-493.

Badr E A, Zahran A A, Cashmore M. Benchmarking performance: Environmental impact statements in Egypt. Environmental Impact Assessment Review 2011;31:279-285.

Canelas L, Almansa P, Merchan M, Cifuentes P. Quality of environmental impact statements in Portugal and Spain.  Environmental Impact Assessment Review 2005;25:217–225.

Conama – Conselho Nacional do Meio Ambiente (Brasil). Resolusion n° 001 of 1986. Brasília: Diário Oficial da União (CONAMA); 1986.

Cortner H J. Making sciene relevant to environmental policy. Environmental Science and Policy 2000;3:21-30.

Glasson J, Therivel R, Chadwick A. Introduction to Environmental Impact assessment. 3rd Edition. London: Spon Press; 2005.

### Gray I, Edward-Jones G. A review of environmental statements in the British forest sector. Impact Assessment and Project Appraisal 2003;21:303-312.

Jay S, Jones C, Slinn P, Wood C. Environmental impact assessment: Retrospect and prospect. Environmental Impact Assessment Review 2007;27:287-300.

MPF – Ministério Público Federal (Brasil). Deficiências em Estudos de Impacto Ambiental: síntese de uma experiência. Brasília: 4ª Câmara de Coordenação e Revisão, Escola Superior do Ministério Público da União, 2004.

Pinho P, Maia R, Monterroso A. The quality of Portuguese Environmental Impact Studies: the case of small hydropower projects. Environmental Impact Assessment Review 2007;27:189-205.

Prado Filho J F, Souza M P. O Licenciamento Ambiental da mineração no Quadrilátero Ferrífero de Minas Gerais – uma análise da implementação de medidas de controle ambiental formuladas em EIAS/RIMAS. Engenharia Sanitária e Ambiental 2004;9:343-349.

Revita Engenharia S. A. Estudo de Impacto Ambiental (EIA) do Centro de Tratamento e Disposição de Resíduos de Caraguatatuba CTR – Caraguatatuba, SP. 2010.

Sánchez L E. Avaliação de impacto ambiental: conceitos e métodos. São Paulo: Oficina de Textos; 2008.

Waldeck S, Morrison-Saunders A, Annandale D. Effectiveness of non-legal EIA guidance from the perspective of consultants in Western Australia. Impact Assessment and Project Appraisal 2003;21:251–256.

Zubair L. Challenges for environmental impact assessment in Sri Lanka. Environmental Impact Assessment Review 2001;21:469-478.