LIGHT POLLUTION MITIGATION – A PROPOSAL METHODOLOGY

1 – OBJECTIVE

This paper aims to establish a methodology for the control of light pollution, ensuring the success of the sea turtles' life cycles during the implementation of industrial projects in areas adjacent to sea turtle nesting beaches.

2- ABSTRACT

For over 30 years Brazil has run an effective program for the conservation of sea turtles in accordance with the Convention on Biological Diversity and the International Union for Conservation of Nature (IUCN). The remaining feeding, nesting and resting areas are known and considered as priorities for conservation of these endangered animals .

Light pollution is the alteration of natural light levels in the outdoor environment.  Present in virtually all human interventions, photopollution is currently a major threat to the success of the conservation of sea turtles. It causes: a) hatchlings to become disoriented, b) discouragement of female turtles from coming ashore to nest and c) an increased vulnerability to predation.

In Brazil, the government agency responsible for environmental licensing processes requires procedures to maintain natural light and strategies to mitigate photopollution.

Over the last ten years, with the stability of the country's economy, several port infrastructure projects, which support units for oil and gas companies as well as residential condominiums and resorts, have been deployed along the Brazilian coast, often in priority sea turtle conservation  areas.

In order to effectively mitigate photopollution, the use of a specific methodology is proposed to accommodate the implementation of projects adjacent to these remaining sea turtle nesting sites, always giving priority to the adoption of eco-efficient lighting systems. This methodology consists of three phases:

Phase 1: Planning and Design; Phase 2 - Construction; Phase 3: Operation and Maintenance.

KEY WORDS:

Environmental management, photopollution mitigation, sea turtle conservation.

3. INTRODUCTION

Photopollution, also known as Light pollution or luminous pollution, is excessive or obtrusive artificial light at night. The International Dark-Sky Association (IDA) defines light pollution as: Any adverse effect of artificial light including sky glow (the brightening of the night sky over inhabited areas), glare (an excessive brightness which causes visual discomfort), light trespass (light falling where it is not intended, wanted, or needed) and  light clutter (bright, and excessive groupings of light sources, commonly found in over-lit urban areas).

Light pollution is a serious concern for the environment. It impacts endangered wildlife such as sea turtles. Presently, artificial lighting is one of the human stressors among the chief causes of disturbances to the successful conservation of these animals.

The harmful presence of artificial lighting in the environment mainly affects sea turtles in their nesting beaches, but also in the nearby water (LORNE&SALMON 2007). Artificial lighting with an intensity of 0 (zero) to 1(one) lux is disruptive and disorienting for sea turtles (PATIRI at alli, 1992) and can even endanger their survival.

They can also potentially interfere in different stages of the sea turtles’ life cycles because 1) they change the behavior of the nesting females when they select the areas used to dig their nests, 2) they disorient nesting females when they return to the sea and 3) most importantly, they disorient the hatchlings when they crawl to the ocean. Because sea turtle nesting normally occurs at night, the clearest negative effect of light pollution is that nesting females usually do not leave the water on beaches where artificial lighting emanates from buildings, roads, highways and public beachside walkways.

These females are often forced to seek out less appropriate areas, which reduces the possibility of successful nesting. Some authors have observed a reduction in nesting activities in well-lit, urbanized beaches (TALBERT at alli, 1980), thereby limiting the remaining number of nesting areas. Other human stressors also systematically influence changes in the environmental characteristics of these areas, such as increased amounts of human activities around buildings near the beach, originating from tourist complexes, condominiums, recreational areas and sea ports  adjacent to industrial parks.

Hatchlings are usually born at night and head directly towards the ocean.  The presence of artificial lighting near their nests disorients them (LOHMANN at alli, 1990). They may also walk in circles, which facilitates and induces predation by other animals. At dawn, when the first rays of sun hit the sand, the hatchlings die from dehydration. This can result in the death of up to 95% of the hatchlings from a single nest (MCFARLANE, 1963). Hatchlings have already been observed to leave the ocean and return to the illuminated areas (photopolluted areas). Experiments revealed Loggerhead (Caretta caretta) hatchlings that swam toward the direction of the artificial lights (O’HARA, 1980; SALMON&LOHMANN,1989), In Brazil, the last remaining areas where sea turtles are known to reproduce (east and northeast coasts of Brazil) are considered a priority for the preservation of these endangered species.  (MARCOVALDI& MARCOVALDI, 1999); MARCOVALDI at alli, 2005)

A specific legislation was drafted to promulgate federal restricting the presence of artificial light in the main nesting areas. The idea was not only to obtain a legal instrument for that purpose but also to provide legal means of regulating future lighting projects in the vicinity of priority areas considered critical for sea turtle protection and management.

In 1990, The National Institute for the Environment and Renewable Natural Resources - IBAMA issued Edict no. 1.933, banning the installation of any artificial light sources directed toward the beach on 63 different Brazilian beaches.

Following more practical experience, this Edict was replaced in 1995 by Edict no. 11, which included more beaches and established specific standards for lighting levels. As a result, a zone excluding artificial light was established. The new law banned the installation of any light source that is greater than ZERO LUX in intensity 50 meters above the high tide in nesting areas.

For the proposed enterprises in these areas (e.g. industrial complexes and ports, resorts, condominiums) the environmental licensing organizations establish specific criteria for restriction and control of photopollution before the initial phase of construction and also during the operating phase with potential penalties of suspension and/or prohibition.

4. METHODOLOGY

A specific methodology was developed in convergence with the requirements of environmental organizations with the objective of mitigating environmental photopollution during all phases of the establishment of an enterprise (project conception, construction, and operation).

The methodology consists of the adoption of specific criteria from the conception of the lighting projects and of the complimentary business operational process (business norms), which are elaborated specifically for each enterprise so that the illumination level in their respective areas of direct influence are compatible with the requirements for conservation of sea turtles.

This methodology is divided into three phases, with some overlaps and interactions among them:

- Phase  1 – PLANNING AND DESIGN

- Phase  2 – CONSTRUCTION (BUILDING PHASE)

- Phase 3 –– OPERATION AND MAINTENANCE

- Phase 1: PLANNING AND DESIGN

At this stage the methodology seeks to educate the entrepreneur and project designer about the environmental sensitivity of the intervening area, seeking the development of specific criteria and of sustainable and eco-friendly projects in order to accommodate the implementation of these projects with the environmental sensitivity of the immediate surroundings, aiming to reduce as much as possible interference in the life cycle of sea turtles;

In this initial phase the process is conceived, for example the description of infrastructure characteristics (height of structures, equipment to be installed), the details of the operational processes and the respective needs of the type of illumination and light intensity. The topography of the region, the proximity to areas of sea turtle reproduction, including their immediate surroundings, are also considered.

With these considerations there are proposed guidelines and criteria for the illumination, and aspects that contribute to environmental photopollution should also be considered:

Sky Glow: Any contribution to the azimuth, whether it be direct focus or reflected light, will encounter clouds and particles of water vapor in suspension, which will serve as a reflecting surface of those contributions, which will return to the surface, including on the beaches where sea turtles lay their eggs.

Direct focus: (Glare and light trespass). Direct focus represents the direct contribution of the actual light bulb on the beach region.

This preliminary information guides the criteria that will be used for the conception of lighting systems, which should consider the following aspects:

- Luminaires: Search for eco-efficient lanterns that provide high return and for a balanced proportion between illumination capacity and energy consumption.

- Types of Lamps: The use of lamps with adequate reproduction of color for the activity and principally with the greatest energy efficiency is recommended.

- Lamp Power: Aim to restrict the power of the bright spot.

- Height of Erected Luminous Spot: Criteria that establishes permitted heights for the supports of the luminaires.

- Confinement of Luminous flux: For luminaires that have dispersions of light movement, installation of light screens is recommended.

- Phase 2 - CONSTRUCTION

At this stage, the methodology seeks the adoption of criteria for construction execution consistent with the environmental sensitivity of the area surrounding the project currently under implementation. The regular monitor (environmental expert) is also expected to be present in order to verify the non-conformities and, when necessary, to propose alternatives and effective solutions. The maintenance teams are trained so that questions can be clarified and to increase the possibilities to effectively regularize the non-conformities.

Disagreements may frequently exist between what is being effectively implemented and the established criteria, given that these works possess very intense event dynamics.

Through regular monitoring, by measuring light intensity with a sensitive luxmeter, adjustments in the criteria discussed above are put into effect, in many cases experimentally given that there sometimes exist difficulties of computer simulation or analytical calculations.

- Phase 3 – OPERATION AND MAINTENANCE

At this stage the proposed methodology outlines criteria for operation and maintenance compatible with the environmental sensitivity of the region, aiming to reconcile the operation of

the projects with the conservation of sea turtles.

This last phase begins at the end of the work of implementing the lighting systems and it is one more phase to accompany operation of the site.

In this phase the adequacy of the level of light is verified according to the activities that are being developed, performing adjustments when considered necessary.

This monitoring is performed regularly and always during the week of the new moon in order not to consider any contribution of the moon in the measurements.

If the enterprise is located in an area near a beach where sea turtles lay their eggs, special attention should be given to the beach line, where it is necessary to consider measurements  while monitoring.

5. FINAL CONSIDERATIONS

This methodology has been used for about five years in different projects with very effective and satisfactory results for the registration and appearance of sea turtles in the project areas. The continuous monitoring carried out by the environmental authorities show that the environmental constraints have been respected in terms of light pollution mitigation. At the same time, the strategy developed does not impair the companies’ activities.

The operational processes of the enterprises should simultaneously attend to good practices for security and employee health, and in the natural environment highlight the environmental sensibility of the enterprises, which vary.

Nevertheless, the eventual impacts of the activities as well as the effective control of environmental photopollution should be checked and evaluated in the area of direct influence of each enterprise.

In the case of non-conformities these processes should be revisited collectively by the environmental, security and productivity teams, and if necessary should be adapted to incorporate eco-efficient questions to reduce the impact on the immediate surroundings of the enterprises, leaving the sea turtle nesting areas as close as possible to the original conditions of light before the implementation of the enterprise.

Complimentarily, environmental education initiatives as well as corporate communications are also encouraged to clarify doubts and to promote environmental awareness among employees and stakeholders in an attempt to achieve success in the challenge of conservation of these endangered animals.

6 - BIBLIOGRAPHICAL REFERENCES

International Dark-Sky Association (IDA) Glossary: light pollution, sky glow, glare; light trespass  and  light clutter. Available:  http://www.darksky.org.  Accessed: March, 27 2012.

LOHMANN, K. J.,SALMON, M. , WYNEKEN,  J. 1990. Functional Autonomy of Land and Sea Orientation Systems in Sea Turtle Hatchlings. Biological Bulletin 179:214–218.

LORNE, JACQUELYN KAY, SALMON, MICHAEL. Effects of exposure to artificial lighting on orientation of hatchling sea turtles on the beach and in the ocean. Endangered Species Research, vol. 3: 23–30, 2007. Penryn, uk

McFARLANE, R.W. 1963. Disorientation of logger-head hatchlings by artificial road lighting. Copeia [S.I.], United States, 1963, pp. 1-153

MARCOVALDI, Maria Angela; MARCOVALDI, Guy. Marine turtles of Brazil: the history and structure of Projeto TAMAR-IBAMA. Biological Conservation, [S.I.], v. 91, 1999, pp. 35-41.

MARCOVALDI, Maria Angela, PATIRI, Victor., THOMÉ João Carlos.  Projeto TAMAR-IBAMA: 24 Years of a Community-Based Conservation Program to Protect Brazilian Sea Turtles  In: Maritime Studies – MAST.  Special Issue 3(2): Sea Turtles as a Flagship Species – Frazier Jack (Org)  Chapter 2, Amsterdam, Netherlands, 2005 , pp.39-62.

O’HARA, J. 1980. Thermal Influences on the Swimming Speed of Loggerhead Turtle Hatchlings. Copeia 1980:773–780.

SALMON M, LOHMANN, KJ .1989. Orientation Cues Used by Hatchling Loggerhead Sea Turtles (Caretta caretta, L.) during offshore migration. Ethology 83:215–228.

PATIRI, Victor José de Andrade; FERREIRA, Sandra Neusa Marchesini e MOREIRA Fº., Godofredo Carneiro. A Influência da iluminação artificial na reprodução das tartarugas marinhas. XI Seminário Nacional de Distribuição de Energia Elétrica. SENDI, Blumenau, 1992, pp. 1-20.

TALBERT, O.R., Jr., S.E.STANCYK, J.M.DEAN and J.M.WILL 1980. Nesting activity of the logger-head turtle (Caretta caretta) in South Carolina. I. A rookery in transition. Copeia [S.l.], United States, 1963, 1980, p. 709-719.