Master Plan for Energy Resources in Iceland

Abstract

The work for preparing a Master Plan for Hydro and Geothermal Energy Resources in Iceland was initiated by the Government of Iceland in 1999.  In the first phase, which was finished in 2003, 19 hydropower options and 24 geothermal options were evaluated and ranked.  In the second phase, which started in 2007 and is now almost completed, 28 hydropower options and 38 geothermal options were evaluated and ranked.

The National Energy Authority and power companies compiled reports on project proposals they wished to have evaluated.  Subsequently, the reports were evaluated and graded by four working groups of experts.  The groups finding were then compiled by a Steering Committee and all projects graded on the basis of their overall feasibility.  Finally, the Steering Committee categorized the project proposals and identified on one hand, proposals that appeared to be feasible in terms of both their economic implications and their environmental consequences, and on the other hand proposals that for economic or environmental reasons should not be carried out, whether for the foreseeable future or only until further impact studies have been made.

The proposed master plan was advertised in August 2011 in accordance with the SEA legislation which was implemented in Iceland in 2006.  More than 200 comments were received from individuals, non-governmental organizations (NGOs) and power companies.  Herein, the main results of the master plan are presented and discussed.

Introduction

Iceland is very rich in renewable energy resources for electricity generation.  The potential generating hydropower capacity has been estimated to be 55 TWh/yr of which 12.3 TWh/yr have been harnessed by 2010.  The potential generating geothermal capacity has been estimated to be 59 TWh/yr of which 4.6 TWh/yr have been harnessed by 2010.

Earlier, energy developments in Iceland were focused on meeting the basics energy needs of the society for space heating and electricity for the general market.  With the construction of the first primary aluminium plant and Búrfell Hydroelectric Project in 1972 and the subsequent further development of heavy industry in Iceland the electricity needed for the general market gradually became relatively small compared to the electricity needed for the heavy industry.  By 2010 only 20% of the electricity generated in Iceland was used by the general market and 80% were used for heavy industry (mainly aluminium production).

As all stationary energy use in Iceland is by renewable energy, development of renewable energy projects in Iceland is not driven by the need to replace non-sustainable energy projects that utilize coal and oil.  The projects are built in order to maximize results from renewable energy sources with sustainability, value creation and efficiency as guiding principles.  This fact has made it very apparent that the utilization of energy resources must take into account not only the energy needs and the economic aspects of the development, but also a range of other interests as well.  This includes other use of land and the environmental impact of the project.

The EIA legislation was implemented in Iceland in 1994.  As discussed by Thórhallsdóttir (2007b) some practitioners feel it has failed to live up to its potential and have criticized it for not coping with uncertainty and cumulative effects, for failing to adequately consider alternatives, and for ultimately failing to affect decision making.  The SEA legislation was implemented in Iceland in 2006.  The main advantage of SEA is that it addresses the cumulative impact of many possible power projects.

Master Plan

The Master Plan was jointly initiated by The Ministry of Industry and The Ministry for the Environment, which share responsibility for the project. At the start of the project, the Ministry of Industry, in co-operation with the Ministry of the Environment, established special Steering Committees to lead the programme.  The first committee, comprising 16 members, prepared a preliminary ranking at the end of 2003, but was unable to complete the final evaluation due to a lack of scientific research. In 2007, a second Steering Committee of 14 members was formed, in order to complete the programme.  In their functions, the Committees have been supported by a number of experts and professionals working in four different working groups.

- Working Group I evaluates what impact proposed power projects will have on nature, the landscape, geological formations, vegetative cover, and flora and fauna, as well as cultural heritage and ancient monuments.

- Working Group II evaluates the impact on tourism, outdoor actvities agriculture, re-vegetation, fishing in rivers and lakes, and hunting.

- Working Group III evaluates the impact that proposed power projects can have on economic activity, employment, and regional development.

- Working Group IV identifies potential power projects, both hydro and geothermal, and carries out technical and economic evaluations.

Each working group graded and ranked the proposed power projects.  Finally, the Steering Committee categorized the project proposals and identified on one hand, proposals that appeared to be feasible in terms of both their economic implications and their environmental consequences, and on the other hand proposals that for economic or environmental reasons should not be carried out, whether for the foreseeable future or only until further impact studies have been made.  The proposed Master Plan for Energy Resources in Iceland was advertised in August 2011 in accordance with the SEA legislation. The main results are summarized in Table 1 and Figure 1.  More than 200 comments were received from individuals, non-governmental organizations (NGOs) and power companies.  Presently, the master plan is being discussed within the government and finally it has to be approved by the parliament.

Discussion and conclusions

Due to the controversial nature of large power projects the Master Plan for Hydro and Geothermal Energy Resources in Iceland is a major step forward in reaching a broad consensus among the nation which power projects should proceed and which power projects should not be carried out, whether for the foreseeable future or only until further impact studies have been made.  According to the proposed Master Plan it is possible to double the electricity generation in Iceland (Table 1 and Figure 1).  Whereas, today the majority of the electricity in Iceland is generated by hydropower the future projects are mainly geothermal power projects.

When the Steering Committee completed their ranking of the power projects in 2011 their ranking was only based on the combined results of working group 1 and 2.  The reason being, that there was a high correlation between the results of working group 1 (effect on nature) and 2 (effect on tourism) but no correlation between the results of other groups (effect on society and economical effects).  According to the law defining the framework for the Master Plan the Master Plan is to be reevaluated every four years.  It is important that the next four years will be used to develop some cost-benefit analysis process which can be used to create ranking of the power projects based on the combined the results of all four working groups. The importance of this is apparent due to the fact that the development of renewable energy projects in Iceland is not driven by the need to replace non-sustainable energy projects that utilize coal and oil.  The projects are built in order to maximize results from renewable energy sources with sustainability, value creation and efficiency as guiding principles.

If, within the Master Plan, the different power projects are appropriately ranked it is assumed that the main debate in Iceland about which power projects should be developed will be within the framework of the associated SEA legislation.  The focus of the EIA for future power projects will then shift from that question towards optimizing the layout and mitigation measures of the projects, accounting for different environmental, economical and social aspects.

References

Steingrímsson, B., Björnsson, S. and Aðalsteinsson, H. (2007):  Master Plan for Geothermal and Hydropower Development in Iceland. A Presentation at a Short Course on Geothermal Development in Central America – Resource Assessment and Environmental Management, organized by UNU-GTP and LaGeo, in San Salvador, El Salvador, November 25th – December 1st, 2007.

Thórhallsdóttir, Th.E, (2007a): Environment and energy in Iceland: A comparative analysis of values and impacts.  Environmental Impact Assessment Review 27 (2007) 522-544.

Thórhallsdóttir, Th.E, (2007b): Stratigic planning at the national level: Evaluating and ranking energy projects by environmental impact.  Environmental Impact Assessment Review 27 (2007a) 545-568.

Homepage for the Master Plan for Hydro- and Geothermal Energy Resources in Iceland. http://www.rammaaaetlun.is/english

Homepage for the Energy Policy for Iceland.  http://www.nea.is/orkustefnunefnd.

Table 1.  Potential and present electricity generation in Iceland

|  | Hydropower | Geothermal | Total |
| --- | --- | --- | --- |
|  | GWh/yr | GWh/yr | GWh/yr |
| Already harnessed (A) | 12.300 | 4.600 | 16.900 |
| Being built 2011 (B) | 585 | 738 | 1.323 |
| Future power projects included in the master plan for energy resources |  |  |  |
| To be harnessed (C) | 2.741 | 9.170 | 11.911 |
| Further studies needed (D) | 6.008 | 3.098 | 9.106 |
| To be protected | 7.280 | 4.059 | 11.339 |
| Total | 16.029 | 16.327 | 32.356 |
| Within protected areas | 465 | 13.616 | 14.081 |
| Possible to harness (A+B+C) | 15.626 | 14.508 | 30.134 |
| Possible to harness (A+B+C+D/2) | 18.630 | 16.057 | 34.687 |

Figure 1.  Potential and present electricity generation in Iceland