Modification of EIA Regulation in Chile Related to HIA

1. Introduction

The Chilean Environmental Impact Assessment System (SEIA) is an integrated procedure that attempts to cover many types of impacts in a single process. The following are the types of impacts that should be addressed, according to current legislation:

a)	Risk to human health due to effluents, emissions or waste

b)	Significant adverse effects on renewable natural resources, including soil, water and air.

c)	Resettlement or significant alteration of the ways of life and customs of human groups.

d)	Proximity to population, protected resources, or areas likely to be affected, as well as the environmental value of the territory where the project will be located;

e)	Significant effects on touristic value or landscape value of an area; and

f)	Effects on monuments, sites of (with?) anthropological, archeological, or historical value, and, in general, those that belong to cultural heritage.

Therefore, EIA in Chile combines Environmental Impact Assessment (EIA), Health Impact Assessment (HIA), and Social Impact Assessment (SIA) in a single process. This integrated approach might cause that not every issue is properly considered or receives adequate attention, especially risk to human health.

2. Current situation

People´s perception of EIA has become more negative over the years for many reasons but mainly because of the political influence on the process. The Government has openly backup certain energy projects that were considered necessary for the country´s development, such us thermoelectric and hydroelectric power plants, influencing not only the final decision on the EIA but also the quality of the information presented by the proponents to EIA. For example, it is well known the Government influence on the consent granted to Ralco EIA (a hydroelectric power plant that required relocating indigenous communities).

People are also skeptical EIA is able to prevent significant environmental effects, mainly because the lack of established methodologies and uniform criteria of assessment.

In 2010 an event came to reinforce people´s bad perception on thermoelectric power and EIA; after a thermoelectric plant’s EIA was approved, and consent was granted to it, the President himself asked the proponent to desist of the project because of the potential effects on a wildlife sanctuary (Punta de Choros) located 25 km from the project, petition that was accepted. People wondered: Did not EIA aims to prevent just that kind of effects? As a result, credibility on EIA reached an all-time low. No new mega thermoelectric power plant has been submitted for assessment since then (Figure 1).

Figure 1: EIA Studies of thermoelectric power plants submitted to SEIA from 2000 to 2011, in terms of number of projects and total power they represent. No EIA statements have been included. The years correspond to the submission year. The power that was rejected or desisted by the proponents is also shown. No Studies were submitted during 2010 and 2011.

The consents granted to thermoelectric plants in highly populated areas has put the focus of attention on health risks and the way they are addressed in EIA, specially synergic effects.

3. Health Impact Assessment in the SEIA

Chilean legislation (Law 19,300) provides that to assess risk to human health it should be considered:

- Primary quality standards in force

- Emission standards in force

- In the lack (absence) of such standards, those in force in other countries should be considered as reference.

Although those are not the only criteria that could be considered, in practice they are the only ones used so far.

Only impacts to human health caused by exposure to pollutants emitted by the project can be considered in EIA.

a) Primary quality standards

Legally, individuals or single sources are not obligated to comply with quality standards to be approved. The reason is that environment quality is affected by a number of sources, not one specific project. The State is the responsible and should comply with quality standards by implementing prevention and decontamination plans.

However, primary quality standards are used as criteria to establish if a project is likely to produce risk to human health, and in most EIAs are the only criteria, especially for air quality assessments. Reference standards from other countries are seldom used, although current legislation allows it.

To assess risk to human health due to air emissions, primary quality standards are compared with the pollutant concentrations predicted by air quality models at the point of exposure. During the environmental assessment process, discussion always revolves around what model was used and its adequacy, or the sufficiency of meteorological data used.

Usually the uncertainty of the results is not discussed, nor is it incorporated in the decision-making process. Basically, if concentrations estimated by a model at the point of exposure plus baseline concentrations are less than the quality standard, then the project is considered not to produce risk to human health, even if it less by only 1% of the standard.

A recent study that reviewed around 100 projects that were granted consent during the last 10 years concluded that there is no guidelines regarding the type of model used, the number of meteorological stations used, the monitoring period used to establish the baseline, the domain resolution, and so on. Every modeler uses different criteria, and it is not even required to show the input and output model files. Under this scenario, there is a reasonable doubt about the way modeling has been performed in EIA. Therefore, the same doubt arises on the way risk health has been approached.

b) Emission standards

Even Law 19,300 provides that emission standards should be consider to asses risk to human health, technically they must not be considered as the only criteria, especially when emission standards are applicable to the entire country, with no distinction of particular conditions of different places. Health impact assessment should take into account the particular characteristics of the project, baseline, topography, meteorology, and existing sources; therefore, it is clear that such risk might exist even when a project meets emission standards.

Until recently, Chile did not have emission standards for thermoelectric plants. Even though the possibility of using international emission standards in EIA has existed since EIA was first implemented in Chile 16 years ago, never an emission standard from other country has been used when assessing a thermoelectric plant.

An emission standard has recently been enacted for thermoelectric plants in Chile, applicable to the whole country. Most probably projects proponents are going to argue that meeting the new emission standard is enough evidence to demonstrate that there is no risk to human health. There is going to be controversy when, as a result of the EIA process, a consent be granted with an emission limit lower than the emission standard in force.

c) Standards from other countries

Law establishes that when there is no primary quality or emission standard available in the country, standards from other countries should be considered. However, standards from other countries are seldom used, because they are not readily available and there is a language barrier in most cases.

4. New Regulations

Since modification of 2010, Law 19,300 provides that the Environmental Assessment Service (SEA) of Chile must establish uniform criteria for EIA through the publication of Guidelines. Since then SEA has been working on a series of publications, from how to properly describe a project, methodologies to describe the baselines, how to predict impacts, and so on.

Another new provision of Law 19,300 is that when there is not quality standard for a pollutant emitted by a project, neither in Chile nor in other countries, EIA must include a specific chapter on potential health risk the project might produce. Reference values (such as those from WHO, ATSDR, among others) should be used as reference. Since most air pollutants are not regulated, this new provision allows assessing the effects of not regulated pollutants.

If reference values are not readily available, they probably will not be used for EIA, just like it happened with international standards.

5. Challenges

Regarding the use of primary quality standards, air quality models should be used under uniform criteria which have not been established so far in Chile. The Environmental Assessment Service of Chile has been working on Guidelines to set minimum requirements when using models, such as the use of a meteorological model (WRF) when there are not enough meteorological observations in a place. Also, these Guidelines will help decide, using objective criteria, when to use a Gaussian model, such as Aermod, and when to use a “puff” model, such as Calpuff. The challenge for SEA on this matter is to build capacity among its professionals to be able to review and object a modeling when appropriate.

There is clearly a flaw in Law 19,300 regarding consideration of emission standards to assess health impacts, which was not corrected in 2010 when it was modifies. However, there are enough technical arguments to interpret this provision as not exclusive of other considerations. SEA attempts to include this in the new Regulation of Law 19,300, although it might be objected by the Attorney General.

Even though international emission standards should not be the only criteria to consider, they help to evaluate the kind of technology a project is proposing. They should be used in that sense, but they are not readily available, neither are international primary quality standards.  To overcome these difficulties, SEA is compiling environmental standards from different countries, to make them available to the public to use them as reference in EIA. Also, SEA is compiling reference values for exposure, for pollutants with no standards.

Hopefully, modification of Law 19,300, the new Regulation of the Law, and publishing Guidelines will help improve EIA in Chile and increase public credibility in EIA.

Germany, Argentina, Australia, Brazil, Canada, Spain, Mexico, United States, New Zealand, the Netherlands, Italy, Japan, Sweden, and Switzerland.