Research of Strategic Environmental Impact Assessment Method on Linear-Project-Plan in Korea

- Applications of The Road Maintenance and Improvement Basic Plan-

Ⅰ. Introduction

Korea, to consider environment on development, faces fulfillment of strategic environment effect assessment which is the system reviewing the results that policy plan can influence environment systematically.

Strategic environmental effect assessment, when establishing a plan, considering environment from the stage of upper decision making and collecting opinions among the interested parties, by previously reviewing the environmental effect which can appear in the business level of the lower stage in the stage of policy plan and development basic plan, fortifying prevention, which is the purpose of environmental assessment, is the system inducing sustainable development through the harmony of development and environmental conservation.

'A road maintenance and improvement basic plan' handled in the study is the most significant plan relating to the road construction and it is the leaner plan aiming at the whole nation, which can greatly influence environment across nation. Hence, by fulfilling the strategic environmental effect assessment in the stage of plan establishment, it has to minimize negative effect which can influence environment. Therefore, the study, after calculating grade of environmental effect through plan intensity by line and space resistance, is to suggest measures to minimize environmental damage on the line with somehow high environmental effect.

Also, since each road line included in the plan is in the stage of decision making different each other, new environmental effect assessment method reflecting such characteristics to be developed and by applying new assessment method, environmental effect of each individual road line is estimated. According to the assessment result, on the lines having environmental issues, route change or the method of reducing like elimination which can be fulfilled in the stage of fulfillment design and basic design stage equivalent to the lower stage are to be suggested. At last, it is to discuss what roles the environmental effect assessment result play to consider the environment of a road maintenance and improvement basic plan.

Ⅱ. Research contents and method

Spatial range of the study was aiming at the whole extent of nation including road routes and routes of analysis target was determined as the whole nation's road routes which is to newly built or expanded in 'a road maintenance and improvement basic plan'. Time range is from 2011 which is the base year of 'a road maintenance and improvement basic plan' to 2020.

Also, items of assessment review on the linear project and method of range set-up is to induce main review items like <Table 1> by mutual comparison(Matrix) of environmental effect elements and environmental fields.

<Table 1> Set the items of assessment review

| Environmental field Environmental effect element |  |  | Water Quality |  | Land and soil |  | Ecology |  | Living Environment |  | Socio- economic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Water | Hydro-logy | Land Use | Geol-ogical topog-raphy | flora and fauna | noise | Land-scapes | Popul-ation | Resid-ential break | Indus-try |
| Policy plan | Road const-ruction plan | Roads (Shape and density) |  |  |  |  | ◯ | ◯ |  |  |  |  |
|  | Road Master Plan | Road extension, disconnection / double track, route location | ◯ | ◯ | ◯ | ◯ | ◯ | ◯ |  | ◯ | ◯ | ◯ |
| The importance of environmental field |  |  | 1 | 1 | 1 | 1 | 2 | 2 |  | 1 | 1 | 1 |
| Legend |  |  | ◯ : Significant adverse effects expected 1 : Low importance of the environment entry 2 : High importance of the environment entry |  |  |  |  |  |  |  |  |  |

In the result of mutual comparison(Matrix), the review items which are expected to have a great effect on the environment were drawn as flora and fauna, and noise field, which was by the characteristics of the road maintenance and improvement basic plan of linear project and fragmentization of ecosystem by road construction, civil appeals of noise and vibration in the surrounding area, and etc were included. To assess items of reviewing target, the study classified the degree of effect by fulfilling ERE(Environmental Risk Estimation) by route and on the high grade meaning route of big effect, was to establish reducing measure.

Method of ERE calculated grades of environmental effect by summing project intensity of road influencing the fauna and flora, and noise field drawn as reviewing items and space resistance of impact zone surrounding road route of which method is like following.

1) Project intensity set-up

Project intensity of road is to show the recognition of somehow environmental effect element by expansion of road which is linear project, single/double line, and etc and since there is difference in the degree of residents' damages by the noise and effect influencing ecosystem along with these factors, is to give scores on this. Integrity score of road project is classified as 5 stages like <Table 2>.

<Table 2> Set the grade of Project intensity

| Extension of road(m) | Grade of intensity | Score of intensity |
| --- | --- | --- |
| 0.1∼25000.0 | Very Low | 1 |
| 25000.1∼50000.0 | Low | 2 |
| 50000.1∼75000.0 | Medium | 3 |
| 75000.1∼100000.0 | High | 4 |
| More than 100000.1 | Very High | 5 |

2) Calculation of space resistance

Space resistance of road is the process judging damage of analysis target area by road project, and the study, considering characteristics of linear project, to calculate score of space resistance, set up the range of impact zone. Range of impact zone, in accordance with the stage of decision-making of road route, since as it is the upper decision making stage uncertainty of route increases, set up range of impact zone like <Picture 1>.

<Picture 1> Range of impact zone, in accordance with the stage of decision-making

And to see the effect on ecosystem, area of 11 conservation areas were calculated and to evaluate the effect of noise and vibration on local residents, areas of residential, industrial, and commercial regions included within impact zone were calculated. Final grade of space resistance was calculated through area ratio of conversation worth zone and residential, commercial, and industrial regions included in impact zone in contradistinction to area of impact zone, and considering calculated area, grade of conservation worth zone and classifying like <Picture 2>, and summing all scores of space resistance of residential, commercial, and industrial regions and 11 conservation areas included within impact zone of each route, total score of space resistance by each route was calculated.

| Importance grade of conservation areas/urban areas |  | The ratio of conserva- tion areas and urban areas within the road impact zone |  | The grades and scores of resistance |
| --- | --- | --- | --- | --- |
| Very Low(I)～Very High(V) |  | 0% |  | No impact(0)/0 |
| Very Low(I) |  | 100% |  | Very Low(I)/1 |
| Low(II) |  | <20% |  | Very Low(I)/1 |
| Low(II) |  | ≥ 20 < 60% |  | Low(II)/2 |
| Low(II) |  | ≥ 60% |  | Medium(III)/3 |
| Medium(III) |  | <20% |  | Low(II)/2 |
| Medium(III) |  | ≥ 20 < 60% |  | Medium(III)/3 |
| Medium(III) |  | ≥ 60% |  | High(IV)/4 |
| High(IV) |  | <20% |  | Medium(III)/3 |
| High(IV) |  | ≥ 20 < 40% |  | High(IV)/4 |
| High(IV) |  | ≥ 40% |  | Very High(V/5) |
| Very High(V) |  | <5% |  | Medium(III)/3 |
| Very High(V) |  | 6-20% |  | High(IV)/4 |
| Very High(V) |  | ≥ 20% |  | Very High(V)/5 |

<Picture 2> The grades and scores of resistance

Final grade of environmental effect assessment was calculated by classifying the sum of project intensity score of each road route and score of space resistance as 5 stages.

Ⅲ. Research result

The study fulfilled environmental effect assessment on individual route using GIS. Environmental effect assessment means that the bigger is the damaged area, the bigger is the effect on the environment, was evaluated with the method like below, and the assessment result is like following.

| Analysis conservation areas and urban areas within the road impact zone by GIS | Ratio of damaged area within the road impact zone |  |
| --- | --- | --- |
| Intensity : Grade Ⅱ (Low) | Space resistance : Grade Ⅲ (Medium) |  |
| Grade of environmental effect assessment : Grade Ⅲ (Medium) |  |  |

In the result of analyzing the effect of current network of roads of expressway on the conservation worth zone of surrounding area and life zone, in 33 road routes suggested in the road maintenance and improvement basic plan, among 30 routes except 3 routes added after assessment, IV and V grades expected to have environmental problems were counted as 4.

|  | Grade of environmental effect assessment |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Ⅰ | Ⅱ | Ⅲ | Ⅳ | Ⅴ |
| Route | 0 | 11 | 15 | 4 | 0 |

Ⅳ. Conclusion and proposal

In establishing the road maintenance and improvement basic plan having close relation with the national life, as the need of harmony in the development and environment stands out, as the method to meet climate and environmental-friendly policy paradigm shift, strategic environmental effect assessment was fulfilled. In the result of mutual comparison (Matrix), environmental effect when the roads are constructed was to be estimated through the assessment on the fauna and flora, and noise field but since strategic environmental effect assessment on the road maintenance and improvement basic plan of linear project aiming at the whole nation had no precedent and had different decision making stage of individual route, new assessment method considering this was developed to apply.

In the result of environmental effect assessment of individual route applying new assessment method, since if routes calculated as IV and V grades meaning big environmental effect are fulfilled only considering economics, not considering environment, it can give big damage on environment, it is considered to get through enough reviewing, and it is required to reconsider on the routes expected to have influence on the wetland conservation area of which importance increases in world-wide even though its final grade is not IV and V grade route and the route passing through Baekdudaegan having a great conservation value in the national level. To prevent roadkill which can occur by the representative linear project like road and railway, it is judged to need prudent reviewing on the routes passing through wildlife preservation area.

Routes expected such environmental effects needs detailed assessment through field investigation and in the case of concerning serious environmental damage, it is suggested to prepare the reliable measure by change project route or delay the project. Plan is fulfilled for the people's convenience and balanced development of land and if reducing method on the negative environmental effect which can occur by the road construction by considering characteristics of road of linear project, strategic environmental effect assessment can be fulfilled corresponding to its original purpose.