SEA, Basin Management and the Water Framework Directive (WFD)

Abstract: In the Southern regions of Portugal and Spain the water basin management plans, established under the Water Framework Directive of the European Union, are extremely important, not only for development and economic purposes but also for landscape, biodiversity and nature conservation. Many water bodies, both superficial and underground, are under stress due to growing pressure from agriculture, agro-industry and urban and industrial consumption. Water scarcity, a present-day serious threat, is due to increase with climate change, and desertification is already a trend in some areas. This paper summarizes three Water Management Plans for the Guadiana, Sado, Mira and Algarve Basins, emphasizing the critical problems, and the measures that are proposed to improve water management in this region. This information is then related to the SEA processes, which are due to contribute for a better understanding and evaluation of future plans and programmes in the region, and which should not ignore the main challenges that the water management imposes, in the short, medium and long term.

Keywords: basin management, river basin management plan, water framework directive, strategic environmental assessment, water bodies

# Introduction

The Water Framework Directive (WFD) adopted in 2000 by the European countries marked a new chapter in the European Water Policy. It establishes a legal framework to protect and restore clean water across Europe and to ensure its long-term and sustainable use setting new environmental objectives for water protection and focusing the water management at the river basin level.

The Directive addresses inland surface waters, transitional waters, coastal waters and groundwater defining 2015 as the deadline for achieving the “good status” for all waters and providing, for the first time at European level, the framework for an integrated management of all water sources. The objectives of good status include, for surface waters, the achievement of both good ecological and chemical status and for groundwaters the achievement of both good chemical and quantitative status. By providing a new overall “good status” objective the WFD coordinates the environmental objectives set in earlier legislation, thus providing the link between emission controls and the achievement of quality objectives. Specific protection zones are also designated in the context of the Directive for the protection of other water uses and objectives (protection of unique and valuable habitats and species, protection of drinking water resources, protection of bathing water) and for those water bodies more stringent requirements can be defined. Another pioneering dimension of the WFD was the introduction of the principle of recovery of costs of water services. Ensuring that the price charged to water consumers reflects the true costs water-pricing policies act as drivers for the efficient use of water resources and, at the same time, contribute to the environmental objectives set for waters by the WFD.

The River Basin Management Plan (RBMP) is the main operational tool for achieving the objectives set for waters within the timescale required by WFD. The RBMP includes a characterization of the River Basin's Districts (RBD), a review of the impact of human activity on the water status, an economic analysis of water use within the river basin that sets the basis for the cost-effectiveness analysis of the various possible measures and allows the establishment of a programme of measures designed to meet the defined environmental objectives.

The public participation in the RBMP preparation is essential for addressing all the social and political concerns and interests related to water issues and to ensure that the necessary measures are adopted. Public participation extends to all water users, to non-governmental organizations and to other stakeholders and groups likely to be affected by the plan. The WFD is implemented through six-year recurring cycles, the first of which covers the period 2009-2015.To date, all European countries have adopted RBMP, with Portugal, Spain and Greece being the only exceptions. In Portugal public consultations are ongoing or have finalized with the RBMPs waiting final approval.

The WFD, as well as “daughter” directives on environmental quality standards, floods and groundwater, represent the only supra-national water arrangements in the world and have initiated a pioneer process of transboundary water management (UNESCO-WWAP, 2012). In a global context of an increasingly demand for water resources the application of principles of the WFD is of core importance to improve water management and to cope with (i) increasingly demands of water for growing populations, (2) increasingly pressures on water sources by human activities and (3) the uncertainty of climate change impacts (floods and drought events and water scarcity).

The WFD has significant interconnections and linkages with other EU legislation, such as the Strategic Environmental Assessment (SEA) directive (2001/42/EC). SEA of the RBMP is the major area of overlap between these directives. In fact, Article 11 of the SEA directive states that coordinated procedures should be developed where there are such overlapping requirements for assessment. To satisfy both directives, these procedures need to include baseline data, assessment of alternatives, mitigation measures, monitoring procedures, and consultation and participation. Once these coordination procedures are in place, then both directives can be expected to ensure that environmental considerations are included in RBMP (Carter & Howe, 2006; Hirji & Davis, 2009). The collection of baseline data for basin characterization, as required by WFD, will provide a bulk of data on water issues to inform requested SEA reports. Likewise any baseline data obtained during previous SEAs could be used to inform river basin characterization procedures. The assessment of alternatives (different RBMP options) during an SEA would ensure compliance with this requirement of WFD. This would provide a broader platform to defend decisions to select particular RBMP options. The suggestion of mitigation procedures involving possible changes to the content of the RBMP could be suggested in the context of SEA thus assisting the achievement of the objectives of the WFD. At last, the development of consultation and public participation procedures – requirements of both directives – could result in a more holistic approach to water resource management (Carter & Howe, 2006).

# Study area

According to WFD requirements eight River Basin Districts (RBD) were identified in Portugal mainland territory (excluding Azores and Madeira archipelagos), three of which are located in the south part of the country: Sado and Mira (RBD 6), Guadiana (RBD 7) and Ribeiras do Algarve (RBD 8). These three regions occupy an area of 27 599 Km2, which represents a third of the total area of the country (89 045 Km2) (INE, 2002; Nemus-Ecossistema-Agro.Ges, 2011a,b; Nemus-Hidromod-Agro.Ges, 2011) (Figure 1). The RBD6 includes both Sado and Mira river basins and the adjacent streams’ watersheds in the Alentejo and Algarve coasts along with the respective groundwater and adjacent coastal waters. The RBD7 integrates the Portuguese part of Guadiana river basin along with the coast streams’ watersheds and the respective groundwater and adjacent coastal waters. The RBD8 includes the streams’ watersheds of the Algarve Region along with the respective groundwater and adjacent coastal waters (Nemus-Ecossistema-Agro.Ges, 2011a,b; Nemus-Hidromod-Agro.Ges, 2011).

Source: GoogleEarth (2012)

Figure 1 – Location of the River Basin Districts studied in the South of Portuguese territory

The total surface area, number of inhabitants, main river basins and number of identified waterbodies by water category (rivers, lakes, transitional and coastal water categories and artificial water bodies in surface waters plus groundwaters) are presented for each RBD in Table 1.

Table 1 – Main characteristics of RBDs of the South of Portuguese territory

| Characteristics |  | RBD6 | RBD7 | RBD8 |
| --- | --- | --- | --- | --- |
| Total area (Km2) |  | 12 149 | 11 611 | 3 839 |
| Population (No.) |  | 345 724 | 200 863 | 409 000 |
| Main River Basins |  |  Alcáçovas  Costeiras entre o Mira e o Barlavento  Costeiras entre o Sado e o Mira  Costeiras entre o Tejo e o Sado  Mira  Roxo  Sado |  Alcarrache  Ardila  Caia  Chança  Cobres  Degebe  Guadiana  Murtega  Xévora |  Barlavento  Arade  Sotavento |
| Surface waters | Rivers | 196 | 227 | 62 |
|  | Lakes (*) | 20 | 20 | 3 |
|  | Transitional | 9 | 5 | 3 |
|  | Coastal | 3 | 2 | 10 |
|  | Artificial | 8 | 6 | 2 |
|  | Total No. | 236 | 260 | 80 |
| Groundwaters |  | 8 | 9 | 23 |

Source: Nemus-Ecossistema-Agro.Ges (2011a,b); Nemus-Hidromod-Agro.Ges (2011)

(*) Reservoirs

Our main objective in this paper is to present the main results of the three RBMP developed for the south region of Portugal (Sado and Mira, Guadiana and Algarve hydrographic regions) by two consortiums leaded by NEMUS, Lda (Nemus-Ecossistema-Agro.Ges and Nemus-Hidromod-Agro.Ges). The significant problems identified and the measures that are proposed to improve water management in the south region are presented and this information is then related to the opportunities of SEA of the RBMP, in one hand, and the opportunities of RBMP for future SEAs of plans and programmes conducted in the region, in the other hand.

# Critical Aspects – surface waters

The status of surface waters (ecological and chemical) was evaluated for the three river basin districts and the pressures responsible for unfavorable water status were identified. According to this diagnosis, a total of 130 surface water bodies in the RBD6 (corresponding to 55 per cent of total number of surface water bodies) are in an unfavorable status. From these 130 water bodies the vast majority (95 per cent) are rivers (i.e. 124) and the remaining six correspond to either reservoirs (i.e. 5) or estuarine water bodies (i.e. 1). All coastal waters and eight of the nine estuarine waters show good (or higher) status. The scenario of the RBD7 is similar. Nearly 56 per cent of surface water bodies (corresponding to 145 surface water bodies) are in an unfavorable status and again nearly 94 per cent of them belong to river category (i.e. 136).  All coastal waters and all the classified estuarine waters of RBD7 show good (or higher) status. The assessment of surface waters’ status in the RBD8 revealed a rather better situation: 51 per cent of total number of surface waters is in a favorable status and 14 per cent were not classified. The remaining 35 per cent show an unfavorable status (moderate, poor or bad) corresponding totally to river water bodies (Nemus-Ecossistema-Agro.Ges, 2011a,b; Nemus-Hidromod-Agro.Ges, 2011). Figure 2 shows the number of water bodies in each final status category for the three RBD.

Source: Nemus-Ecossistema-Agro.Ges (2011a,b); Nemus-Hidromod-Agro.Ges (2011)

Figure 2 – Final status of surface waterbodies

For rivers, which comprise the bulk of water bodies in the river basin district, the main elements indicating that good ecological status are not being achieved are invertebrates, phytobenthos, phosphates and nitrogen. Surface water degradation in the south territory is associated with both diffuse (mainly from agriculture) and point pollution being the high concentrations of phosphorus and nitrogen one of the major causes of non-achievement of good ecological status. Hydromorfological pressures and flow irregularity are other factors responsible for the unfavorable status of waters. Regarding hydromorphological conditions of rivers, it was verified an deficient implementation of environmental flow regimes in all the RBDs along with the interruption of the river continuum due to barriers to fish passage (mainly dams and weirs). Water abstraction without an adequate control coupled with the lack of cleaning and conservation of some water streams, erosion and changes in land use, have introduced significant changes in flow regime, with more significant impacts as lower flow rates are recorded. It is also worth mentioning the impacts associated with abandoned mines, which give rise to acidic runoff enriched with metals, mainly in RBD6 and 7. The Alqueva dam is an important strategic water reserve for RBD7 ensuring flow regulation and surface water availability. However, in the Guadiana river basin there is a significant dependence of flow coming from Spain, which can compromise water availability, a theme that is being fully discussed at the transboundary level (between Spanish and Portuguese authorities) (Nemus-Ecossistema-Agro.Ges, 2011a,b; Nemus-Hidromod-Agro.Ges, 2011).

# Critical Aspects – Groundwaters

The status of groundwater bodies (chemical and quantitative) was evaluated for the three river basin districts and the pressures responsible for unfavorable water status were identified. Figure 3 shows the number of water bodies in each final status category for the three RBD.

Source: Nemus-Ecossistema-Agro.Ges (2011a,b); Nemus-Hidromod-Agro.Ges (2011)

Figure 3 – Final status of groundwater bodies

Regarding RBD6 currently all groundwater bodies are at good quantitative status, 88 per cent are at good chemical status, which means overall that 88 per cent are at good status. Classification of “Sines” waterbody in poor state was due to concentration of organic compounds derived from petroleum associated with industrial pollution. Nevertheless groundwater quality does not affect the quality of surface water nor contribute to the degradation of Groundwater Dependent Terrestrial Ecosystems (Nemus-Ecossistema-Agro.Ges, 2011a,b; Nemus-Hidromod-Agro.Ges, 2011). As regards RBD7 currently 89 per cent of groundwater bodies are at good quantitative status, 67 per cent are at good chemical status, one water body (“Moura-Ficalho”) was not classified which means overall that 56 per cent are at good status. Environmental quality objectives are not met for three groundwater bodies due to nitrates associated with diffuse pressures (“Elvas- Campo Maior”, “Elvas-Vila Boim” and “Gabros de Beja”). From the twenty-three groundwater bodies of RBD8, four were classified as having a poor chemical state (i.e. 17 per cent) mainly due to nitrate concentrations. Nevertheless groundwater quality does not affect the quality of surface water nor contribute to the degradation of Groundwater Dependent Terrestrial Ecosystems. Twenty-two groundwater bodies (96 per cent) are at good quantitative status and only one water body (“Campina de Faro”) remains unclassified relative to quantitative state. Overall 83 per cent are at good status (Nemus-Ecossistema-Agro.Ges, 2011a,b; Nemus-Hidromod-Agro.Ges, 2011).

# Challenges For Water management In the South Region

For the three Plans detailed Programmes of Measures were developed to meet the defined environmental objectives. These integrate basic measures (which include meeting the requirements of other relevant Directives and the licensing of discharges and abstraction) complemented by supplementary and other measures when the basic measures proved to be insufficient to meet the defined environmental objectives. All the three plans set out where water bodies’ good status cannot be achieved by 2015. In these cases an alternative objective of good status by 2021 or 2027 was set. Achieving good status in all water bodies by no more than 2027 in the three river basin districts, as proposed, is a significant challenge. The information gained from research during these first generation plans (2009-2015) will help to accelerate improvement to known issues using both traditional and novel techniques in both second and third cycles (2015-2021 and 2021-2027 respectively).

The pressures in the river basin districts will evolve significantly in some of the main river basins. It’s the case of “Costeiras entre o Sado e Mira” basin in RBD6, of the “Degebe” and “Guadiana” basins in RBD7 and of “Arade” basin in RBD8.  Agriculture will respond to the changed climate, market conditions, financial incentives and regulatory pressures, but the expected tendency is an increase of water demands from this sector. Given this situation and the lack of efficiency of irrigation infrastructures, measures related to the recovery and modernization of public irrigation schemes and promotion of efficient use of water in agriculture were proposed. Furthermore, to ensure water availability for the main economic activities, balance between demand and availability and the need of keeping a reserve volume for ecological requirements were both considered.

Despite some degree of uncertainty associated with estimation of organic, nutrient and hazardous substances pollutant loads to water bodies, qualitative pressures were identified as one of the main causes for unfavorable status of waters. In fact, in some cases it was difficult to identify appropriate remedies for water bodies that are currently not achieving good ecological status. Sometimes this was because the cause of the problem and its sources are not yet known. In other cases the most appropriate solution to the problem needs to be researched. Investigations into these types of issues were important actions proposed in the RBMPs’ Programs of Measures. Opportunities for a better regulation, supervision and monitoring of domestic wastewater discharges as well as discharges from food industry and non-food industry and farming were also identified in the diagnosis of river basin districts. These opportunities led also to the establishment of measures to fulfill these gaps and to ensure that water bodies achieve environmental goals proposed. Besides qualitative pressures and abstraction, interruption of river and habitat continuity, hydrological alterations as well as other pressures (i.e. proliferation of exotic species) were also appointed in the Plans as responsible for unfavorable water bodies’ status. In a general way Programs of Measures include actions directed to the most important pressures that influence the status of water bodies in the studied river basins namely: (1) the control of point source pollution; (2) the control of diffuse pollution; (3) the regulation of water availability to meet all the demands; (4) interventions to deal with hydromorphological changes; and (5) the control of other types of pressures including those resulting from climate change.

SEAs of each RBMP were also performed according to the legal requirements. The SEA process of RBMPs has ensured that potential environmental impacts (both positive and negative) associated with the implementation of the RBMPs have been given due consideration in the preparation of the Plans. The primary effect of the RBMPs will be to improve water quality and ensure good ecological and chemical status by 2015 and beyond, with many of the measures resulting in direct positive impacts on water and aquatic biodiversity as a result. So, in terms of SEA, the three Plans were evaluated as strongly positive and where potential negative impacts have been identified, mitigation measures were proposed to be accomplished in the Plans’ final versions.

# Final considerations

In this paper we presented three RBMPs done by two consortiums leaded by NEMUS, Lda for the south region of Portugal. The elaboration of the first generation RBMPs was a major challenge at the national level due to the pioneering nature of WFD approach. A new legal framework was created compliant with WFD principles, the water sector was redesigned and a huge effort of data compilation and organization was done. Significant advances were observed in terms of harmonization of procedures for status monitoring and assessment and pressures and impacts characterization. Some aspects still need further improvement such as: (1) the capacity of local, regional and national institutions; (2) the evaluation of risk areas (flood, dam failure, accidental pollution); and (3) the knowledge and diagnosis of certain water bodies. SEAs of the RBMP are also essential to establish a clear policy analysis and decision framework to guide decentralized water management

Stakeholder input into the process i.e. feedback Characterisation of RBD Stakeholder involvement Consultation on draft RBMP Monitoring Consultation on SEA of draft RBMP RBMP SEA of draft RBMP SEA of future plans resulting from RBMP RBMP setting the framework for further plans/ projects Note: A dotted line --- shows where linkages/events may occur Adapted from Sheate & Bennett (2007)

Figure 4 – Linkages between WFD and the SEA

The final approval of RBMP and thus the implementation of the designed Programmes of Measures constitute a challenge in regional and national terms, only achievable through a significant effort and a very strong commitment from public authorities and all the stakeholders involved. The data collected in the future SEAs of projects and plans resulting from the RBMPs can be used in the next cycle to make easy the review of RBMP and to feed into the SEA for the second cycle of the RBMP process.

REFERENCES

Sheate, W. R. & Bennett, S. (2007). The Water Framework Directive, Assessment, Participation and Protected Areas: What are the Relationships? (WAPPA) (2005-W-DS-24-M1) Synthesis Report. Environmental Protection Agency, Ireland.

Carter, J.G., Howe, J (2006). The Water Framework Directive and the Strategic Environmental Assessment Directive: exploring the linkages. Environmental Impact Assessment Review 26 (3): 287-300.

Hirji, R. & Davis, R. (2009). Strategic Environmental Assessment: Improving Water Resources Governance and Decision Making. Main Report. Water Sector Board Discussion Paper Series. Paper No. 12, April 2009. The World Bank, Washington, DC.

INE (2003). Anuário Estatístico de Portugal 2002/ Statistical Yearbook of Portugal 2002. Instituto Nacional de Estatística (INE), Portugal. 2003 (Ano de Edição).

NEMUS-ECOSSISTEMA-AGRO.GES (2011a). Plano de Gestão das Bacias Hidrográficas Integradas na Região Hidrográfica 6 (Sado e Mira). Volume I- Relatório (versão preliminar para efeitos de consulta pública). ARH do Alentejo. 2011.

NEMUS-ECOSSISTEMA-AGRO.GES (2011b). Plano de Gestão das Bacias Hidrográficas Integradas na Região Hidrográfica 7 (Guadiana). Volume I- Relatório (versão preliminar para efeitos de consulta pública). ARH do Alentejo. 2011.

NEMUS-HIDROMOD-AGRO.GES (2011). Plano de Gestão das Bacias Hidrográficas Integradas na Região Hidrográfica 8 (Ribeiras do Algarve). Volume I- Relatório (versão preliminar para efeitos de consulta pública). ARH do Algarve. 2011.

UNESCO-WWAP (2012). Management Water Under Uncertainty and Risk.  Overview of Key Messages. United nations World Water Development Report 4 (WWDR4). United Nations World Water Assessment Programme (WWAP), United Nations Educational, Scientific and Cultural Organization (UNESCO). 2012.